Review-1: Research Papers – Real-Time Pedestrian Detection and Collision Avoidance System

| S.No | Name of the paper and Authors | Year of Publication | Downloadable Link | Journal Name / Scopus or Web of Science | Dataset Used | Methodology Used | Limitations |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Pedestrian Detection Using Multispectral Images and a Deep Neural Network Jason Nataprawira; Yanlei Gu; Igor Goncharenko; Shunsuke Kamijo | 2021 | Download PDF | Sensors — Scopus; Web of Science (SCIE) | KAIST multispectral pedestrian dataset | Deep neural network using RGB and thermal/multispectral information for pedestrian detection. | Nighttime and difficult illumination can still reduce detection performance; multispectral sensing requires additional sensor data. |
| 2 | A Pedestrian Detection Algorithm Based on Score Fusion for Multi-LiDAR Systems Tao Wang; Jianhua He; Li Yu; Kai Deng | 2021 | Download PDF | Sensors — Scopus; Web of Science (SCIE) | KITTI 3D object detection dataset + self-built dataset | Two LiDAR detection pipelines with point-density/location features; Bayesian score fusion of classifiers. | Uses multiple LiDAR sensors and a self-built dataset; future work calls for more discriminative/lightweight classifiers and broader sensor combinations. |
| 3 | A Review of Deep Learning-Based Methods for Pedestrian Trajectory Prediction Bogdan Ilie Sighencea; Rareș Ion Stanciu; Cătălin Daniel Căleanu | 2021 | Download PDF | Sensors — Scopus; Web of Science (SCIE) | Review of ETH, UCY, KITTI, Oxford RobotCar, Cityscapes and other datasets | Review/comparison of RNN, CNN and GAN-based pedestrian trajectory prediction methods, sensors and evaluation metrics. | Review study rather than a deployed collision-avoidance system; datasets and sensing conditions are heterogeneous. |
| 4 | Pedestrian and Vehicle Detection in Autonomous Vehicle Perception Systems—A Review Md Nazmul Huda; others | 2021 | Download PDF | Sensors — Scopus; Web of Science (SCIE) | Reviews Caltech, KITTI, PIE, BDD100K and other AV datasets | Review of traditional computer vision and deep-learning pedestrian/vehicle detection methods for autonomous-vehicle perception. | Small, occluded and truncated pedestrians remain challenging; difficult lighting/weather conditions are insufficiently covered by common datasets. |
| 5 | Adopting the YOLOv4 Architecture for Low-Latency Multispectral Pedestrian Detection in Autonomous Driving Kamil Roszyk; Michał R. Nowicki; Piotr Skrzypczyński | 2022 | Download PDF | Sensors — Scopus; Web of Science (SCIE) | KAIST multispectral pedestrian dataset | YOLOv4/YOLOv4-Tiny with RGB-thermal fusion; evaluated for accuracy, latency and false negatives. | Multispectral hardware increases system complexity; performance is still dependent on illumination, alignment and false-negative control. |
| 6 | Research on Pedestrian Detection and DeepSort Tracking in Front of Intelligent Vehicle Based on Deep Learning Xuewen Chen; Yuanpeng Jia; Xiaoqi Tong; Zirou Li | 2022 | Download PDF | Sustainability — Scopus; Web of Science (SCIE) | Intelligent-vehicle/front-view traffic and pedestrian image/video data | YOLO-based pedestrian detection enhanced with channel/spatial attention, followed by DeepSORT tracking. | Small and partially occluded pedestrians can still cause tracking failures; complex crowded scenes remain difficult. |
| 7 | An Improved YOLOX Model and Domain Transfer Strategy for Nighttime Pedestrian and Vehicle Detection Kefu Yi; Kai Luo; Tuo Chen; Rongdong Hu | 2022 | Download PDF | Applied Sciences — Scopus; Web of Science (SCIE) | BDD100K; 4,800 manually selected nighttime images + 9,399 daytime images | Lightweight YOLOX with coordinate attention, feature-scale fusion, CIoU/Varifocal loss and daytime-to-night domain transfer. | Night/fog and other adverse conditions remain challenging; authors identify future need for radar/LiDAR and further fog-condition studies. |
| 8 | Online Trajectory Planning with Reinforcement Learning for Pedestrian Avoidance Árpád Fehér; Szilárd Aradi; Tamás Bécsi | 2022 | Download PDF | Electronics — Scopus; Web of Science (SCIE) | CARLA Simulator scenarios + human-driver comparison | Reinforcement learning for local trajectory planning with MPC and Stanley controllers for longitudinal/lateral control. | Evaluation is largely simulation-based; real-world sensor noise, pedestrian diversity and road complexity are not fully represented. |
| 9 | A Pedestrian Detection Network Model Based on Improved YOLOv5 Ming-Lun Li; Guo-Bing Sun; Jia-Xiang Yu | 2023 | Download PDF | Entropy — Scopus; Web of Science (SCIE) | WiderPerson pedestrian dataset | Lightweight YOLOv5s-G² using Ghost and GhostC3 modules to reduce computation while retaining detection capability. | Lightweight design trades some representational capacity for efficiency; difficult small/occluded targets remain a challenge. |
| 10 | D-STGCN: Dynamic Pedestrian Trajectory Prediction Using Spatio-Temporal Graph Convolutional Networks Bogdan Ilie Sighencea; Ion Rareș Stanciu; Cătălin Daniel Căleanu | 2023 | Download PDF | Electronics — Scopus; Web of Science (SCIE) | ETH, UCY and Stanford Drone Dataset (SDD) | Spatial and temporal graph neural networks with attention plus a time-extrapolator CNN for trajectory prediction. | Trajectory benchmarks are mainly video/surveillance datasets; direct integration with real-time vehicle control and collision braking is not provided. |
| 11 | An Improved YOLOv7 Lightweight Detection Algorithm for Obscured Pedestrians Chang Li; Yiding Wang; Xiaoming Liu | 2023 | Download PDF | Sensors — Scopus; Web of Science (SCIE) | CrowdHuman pedestrian-intensive dataset | YOLOv7 with MobileNetV3 backbone, high-resolution feature pyramid and attention-based detection head. | Designed mainly for image detection; real-world motion prediction and collision-avoidance actions are outside the study scope. |
| 12 | All-Weather Pedestrian Detection Based on Double-Stream Multispectral Network Chih-Hsien Hsia; Hsiao-Chu Peng; Hung-Tse Chan | 2023 | Download PDF | Electronics — Scopus; Web of Science (SCIE) | KAIST and FLIR multispectral pedestrian datasets | Double-stream RGB/thermal YOLO-based detector with multispectral fusion and illumination-aware adaptive weighting. | Multispectral processing can be computationally demanding; cloud/edge computing is suggested for speed, and deployment hardware adds complexity. |
| 13 | Holistic Spatio-Temporal Graph Attention for Trajectory Prediction in Vehicle–Pedestrian Interactions Hesham Alghodhaifi; Sridhar Lakshmanan | 2023 | Download PDF | Sensors — Scopus; Web of Science (SCIE) | VCI-DUT and inD datasets | Spatio-temporal graph attention to model vehicle–pedestrian interactions and predict pedestrian trajectories. | Focuses on prediction rather than the complete detection-to-braking pipeline; dataset availability and generalization to unseen traffic environments remain concerns. |
| 14 | Vehicle–Pedestrian Detection Method Based on Improved YOLOv8 Bo Wang; Yuan-Yuan Li; Weijie Xu; Huawei Wang; Li Hu | 2024 | Download PDF | Electronics — Scopus; Web of Science (SCIE) | Merged KITTI dataset; 7,481 labeled images, train/test split 8:2 | YOLOv8n improved with BiFormer attention, 160×160 small-target head, weighted BiFPN and WIoUv3 loss. | Training/testing uses KITTI-derived data; authors note data availability by request and the model remains primarily a detection system without full collision-avoidance control. |
| 15 | Impact of Perception Errors in Vision-Based Detection and Tracking Pipelines on Pedestrian Trajectory Prediction in Autonomous Driving Systems Wen-Hui Chen; Jiann-Cherng Wu; Yury Davydov; Wei-Chen Yeh; Yu-Chen Lin | 2024 | Download PDF | Sensors — Scopus; Web of Science (SCIE) | Argoverse 1, BDD100K and KITTI; additional real-world test locations | Vision detection/tracking pipelines with trajectory prediction; studies error accumulation from perception into prediction and distance estimation. | Detection/tracking errors propagate into trajectory and distance estimation; paper recommends tighter integration of detection, tracking, prediction and distance estimation. |

PDF links (one per paper):

1. https://www.mdpi.com/1424-8220/21/7/2536/pdf

2. https://www.mdpi.com/1424-8220/21/4/1159/pdf

3. https://www.mdpi.com/1424-8220/21/22/7543/pdf

4. https://www.mdpi.com/1424-8220/21/21/7267/pdf

5. https://www.mdpi.com/1424-8220/22/3/1082/pdf

6. https://www.mdpi.com/2071-1050/14/15/9281/pdf

7. https://www.mdpi.com/2076-3417/12/23/12476/pdf

8. https://www.mdpi.com/2079-9292/11/15/2346/pdf

9. https://www.mdpi.com/1099-4300/25/2/381/pdf

10. https://www.mdpi.com/2079-9292/12/3/611/pdf

11. https://www.mdpi.com/1424-8220/23/13/5912/pdf

12. https://www.mdpi.com/2079-9292/12/10/2312/pdf

13. https://www.mdpi.com/1424-8220/23/17/7361/pdf

14. https://www.mdpi.com/2079-9292/13/11/2149/pdf

15. https://www.mdpi.com/1424-8220/24/15/5066/pdf